13/10/26                                                                                              E. Mahima

192312227

ITA0329 – MOBILE COMPUTING

CO1 – ASSESSMENT TOOL 2

Code of Conduct

I __E. Mahima (192312227)__ certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the Student: __E. Mahima__

Q1.  Full-duplex voice channels

Given: Total spectrum = 45 MHz, simplex channel bandwidth = 25 kHz.

For full-duplex, two simplex channels are required.

N = (45 * 1000) / (2 *25) = 900 channels

Answer: 900 full-duplex channels

Q2.  Channels allocated per cell

N = {Total channels} / {Cluster size} = {560} / {7} = 80

Answer: 80 channels per cell

Q3.  Total GSM voice channels

Total simultaneous voice channels = 100 carriers × 8 time slots = 800 channels.

Answer: 800 simultaneous voice channels

Q4.  Power control ratio in Db

Power ratio = 10 log₁₀(80/4) = 10 log₁₀(20) ≈ 13.01 dB.

Answer: 13.01 dB

Q5.  Carrier-to-interference ratio (C/I)

C/I = Carrier power − Interference power

= −60 − (−92) = 32 dB.

Answer: 32 dB

Q6.  Channels per cell

Given: Spectrum = 42 MHz, channel bandwidth = 300 kHz, cluster size = 7.

Total channels = 42 MHz ÷ 300 kHz = 140.

Channels per cell = 140 ÷ 7 = 20 channels.

Answer: 20 channels per cell

Q7.  Received power at twice the distance

Path-loss exponent (n=4).

With path-loss exponent 4,

doubling distance reduces power by 2⁴ = 16.

Received power = 64 W ÷ 16 = 4 W.

Answer: 4 W

Q8. Transmit power required to double the cell radius

For an inverse-square power law:

transmit power is proportional to radius squared.

Doubling radius requires 2² = 4 times the power.

Answer: 4 times the original power

Q9. Total channels in the cluster

Total channels in the cluster = 4 cells × 15 channels = 60 channels.

Answer: 60 channels

Q10. Doppler shift at 100 km/h and 900 MHz

Convert velocity:

Doppler shift (maximum) = vf/c.

Speed = 100/3.6 = 27.78 m/s,

So, fD = (27.78 × 900 × 10⁶)/(3 × 10⁸) ≈ 83.3 Hz.

Answer: Approximately 83.33 Hz

Q11. Duration of one TDMA time slot

Slot duration = Frame duration ÷ Number of slots

= 5 ms ÷ 8

= 0.625 ms.

Answer: 0.625 ms

Q12. Number of GSM carrier frequencies

Number of carriers = 24 MHz ÷ 200 kHz

= 120 carrier frequencies.

Answer: 120 carrier frequencies

Q13. Received power in dBm

Convert 8 W to dBm:

8 W = 10 log₁₀(8000) ≈ 39.03 dBm.

Received power = 39.03 − 33 = 6.03 dBm.

Answer: 6.03 dBm

Q14. Free-space path loss (FSPL)

Free-space path loss = 32.44 + 20 log₁₀(f in MHz) + 20 log₁₀(d in km)

= 32.44 + 20 log₁₀(1900) + 20 log₁₀(2.5)

≈ 105.97 dB.

Answer: Approximately 106 dB

Q15. Total cluster call-handling capacity

Total call capacity = 6 cells × 250 calls per cell

= 1,500 simultaneous calls.

Answer: 1,500 simultaneous calls

Q16. Number of active calls

Given: 4,000 calls/s, average call duration = 3 minutes = 180 seconds.

Number of active calls = Call arrival rate × Average call duration.

Average duration = 3 minutes = 180 seconds.

Therefore, 4,000 × 180 = 720,000 active calls.

Answer: 720,000 active calls

Q17. Total bandwidth consumed

Total bandwidth = 150 channels × 25 kHz

= 3,750 kHz

= 3.75 MHz.

Answer: 3.75 MHz

Q18. Total guard-band bandwidth wasted

Assuming one guard band between each adjacent pair of 30 frequency bands:

There are 29 guard bands between 30 adjacent frequency bands.

Total guard-band bandwidth = 29 × 40 kHz = 1,160 kHz

= 1.16 MHz.

Answer: 1.16 MHz

Q19. Number of handoffs per hour

Given: Speed = 90 km/h, cell radius = 1.5 km.

Cell diameter:

Cell diameter = 2 × 1.5 = 3 km.

At 90 km/h,

estimated crossings per hour = 90 ÷ 3 = 30 handoffs.

Answer: Approximately 30 handoffs per hour

Q20. Total network user-handling capacity

Total user capacity = 8 regional centers × 40 base stations × 250 users

= 80,000 users.

Answer: 80,000 users

Q21. Number of cells required

Given: Region area = 400 km², radius = 400 m = 0.4 km.

Using the specified approximation,

Radius = 400 m = 0.4 km.

Using the given approximation, cell area ≈ R² = 0.4² = 0.16 km².

Number of cells = 400 ÷ 0.16 = 2,500 cells.

Answer: Approximately 2,500 cells

Q22. Number of FDMA channels

Number of FDMA channels = 6 MHz ÷ 30 kHz

= 200 channels.

Answer: 200 channels

Q23. Offered traffic in Erlangs

Given: 1,200 subscribers, 4 calls/hour each, average call duration = 2.5 minutes.

Traffic formula:

Offered traffic = Subscribers × Calls per hour × Call duration (minutes) ÷ 60

= 1,200 × 4 × 2.5 ÷ 60 = 200 Erlangs.

Answer: 200 Erlangs

Q24. Maximum simultaneous users

Maximum simultaneous users = Total bandwidth ÷ Bandwidth per user

= 40 MHz ÷ 400 kHz

= 40,000 ÷ 400

= 100 users.

Answer: 100 simultaneous users

Q25. Total network user capacity

Total network user capacity = Number of base stations × Users per base station                       = 1,200 × 600 = 720,000 simultaneous users.

Answer: 720,000 users